Autonomous SLAM-Based Mobile Robot

Project Scope, Architecture, Requirements and Development Roadmap

Project type: Long-term personal engineering project
Team size: 2 people
Primary domain: Robotics, IIoT, embedded systems and autonomous navigation
Primary application: Indoor warehouse material transport
Development approach: Simulation-first, followed by physical implementation
Future use: Portfolio, hackathons, technical interviews and potential Semester 7 minor project

1. Project vision

Develop a low-cost autonomous indoor mobile robot that can map an unfamiliar environment, estimate its own position, navigate to selected destinations, detect obstacles and replan its route when necessary.

The robot will communicate with a locally hosted web dashboard over Wi-Fi. The dashboard will provide a live view of the robot's map, position, sensor data, operating mode and navigation status. It will also allow the user to drive the robot manually or select a destination for autonomous navigation.

The project will combine physical robotics with a usable software interface, making it a complete end-to-end system rather than an isolated hardware prototype.

2. Problem statement

Many basic mobile robots follow predefined movements or use proximity sensors to avoid nearby obstacles. Such systems generally do not maintain a complete representation of an unfamiliar environment or estimate their position on a map while navigating.

The proposed system will investigate how a low-cost differential-drive robot can combine 2D LiDAR, wheel encoders and appropriate localization and navigation algorithms to perform indoor autonomous navigation without GPS.

The system will also provide a human-operated interface for monitoring the robot, sending movement commands, selecting navigation goals and viewing system status over a local wireless network.

The solution will initially target a controlled indoor warehouse-like environment and will be evaluated through simulation and physical experiments.

3. Objectives

- Understand and implement the fundamentals of mobile robot kinematics and odometry.

- Develop a simulated indoor environment and validate navigation algorithms.

- Design and manufacture a custom 3D-printed robot chassis.

- Integrate LiDAR, wheel encoders, a motor controller and an onboard computer.

- Generate a 2D occupancy-grid map using SLAM.

- Localize the robot and navigate to user-selected destinations.

- Detect obstacles and stop or replan when a route becomes unsafe.

- Develop a web dashboard for live monitoring and manual/automatic control.

- Evaluate the system quantitatively using repeatable experiments.

- Maintain code, documentation, CAD files and test results for a portfolio-quality project.

4. Defined scope

Core features — required

- Differential-drive mobile robot.

- Custom 3D-printed chassis and sensor mounts.

- Wheel-encoder-based odometry.

- 2D LiDAR integration.

- SLAM-based mapping.

- Localization on the generated map.

- Goal-based autonomous navigation.

- Global path planning using an appropriate planner, initially A* or a compatible navigation-stack planner.

- Obstacle detection and safe stopping.

- Local Wi-Fi communication.

- Live telemetry and robot status on a web dashboard.

- Manual driving using dashboard controls.

- Switching between manual and autonomous modes safely.

- Simulation and physical validation.

Advanced features — only after the core works

- Dynamic obstacle avoidance and automatic replanning.

- Detection and recovery from localization failure.

- IMU-assisted sensor fusion.

- Multiple destination waypoints or delivery tasks.

- Comparison of different SLAM configurations or planning algorithms.

- Improved diagnostics, logging and performance analytics.

Explicitly outside the initial scope

- Outdoor GPS navigation.

- Operation on public roads or unsupervised crowded areas.

- Heavy-payload transport.

- Multi-robot coordination.

- Deep-learning-based object detection.

- Cloud-dependent control or remote internet operation.

- Industrial or medical deployment certification.

These may be considered later if there is a clear technical reason to expand the project.

5. Proposed system architecture

A. Physical robot layer

- Main computer: Raspberry Pi or another compatible Linux computer.

- Sensors: 2D LiDAR and wheel encoders; IMU optional.

- Low-level controller: ESP32 or Arduino.

- Actuation: two geared DC motors, motor driver and differential-drive wheels.

- Supporting hardware: battery, regulators, switch, chassis and mounts.

B. Robotics software layer

- Ubuntu Linux.

- ROS 2.

- LiDAR and encoder drivers.

- Robot description and coordinate transforms.

- Odometry and sensor-data processing.

- SLAM and localization.

- Global and local navigation.

- Velocity control and safety monitoring.

C. Backend and communication layer

- A Python backend using FastAPI or a similar lightweight framework.

- A ROS 2-to-web bridge that publishes selected telemetry and receives validated commands.

- WebSockets for low-latency live updates.

- HTTP endpoints for settings, status queries and selected configuration operations.

- Local Wi-Fi networking between the robot and the user's laptop or phone.

D. Frontend layer

A responsive browser-based dashboard, accessible from a laptop or phone on the same local network.

The dashboard should provide:

- Live map and robot-position display.

- Current operating mode.

- Manual directional controls.

- Start, stop and navigation-goal controls.

- LiDAR or obstacle-distance visualization where practical.

- Battery voltage or estimated battery status, if suitable hardware is available.

- Connection status, errors and event logs.

- Navigation progress and goal-arrival status.

A browser-based interface is preferred over a native mobile app for the first version because it avoids maintaining separate Android, iOS and desktop applications.

6. Proposed hardware requirements

| Component | Priority | Purpose |
| --- | --- | --- |
| Raspberry Pi or compatible Linux computer | Essential | Runs ROS 2, SLAM and navigation |
| 2D LiDAR | Essential | Mapping and obstacle measurements |
| Two encoder-equipped DC geared motors | Essential | Movement and odometry |
| ESP32 or Arduino | Recommended | Motor and encoder interface |
| Compatible motor driver | Essential | Drives the motors |
| Battery and voltage regulation | Essential | Reliable power |
| Custom 3D-printed chassis | Essential | Supports and mounts components |
| Wheels and caster | Essential | Differential-drive movement |
| IMU | Optional | Additional motion measurements |
| Ultrasonic sensors | Optional | Supplementary close-range sensing |
| Camera | Optional | Future visual perception |
| Wi-Fi router or access point | Conditional | Local networking if the existing network is unsuitable |

Before buying anything, inventory the lab's available components and verify their electrical specifications and software compatibility.

The main cost risks are the LiDAR, onboard computer, encoder-equipped motors and reliable power system. Existing lab equipment should be reused wherever possible.

7. Software and tools

| Tool | Purpose |
| --- | --- |
| MATLAB and available toolboxes | Simulation, algorithm experiments and data analysis |
| MATLAB/Simulink robotics capabilities | Modelling and testing where supported by the installed toolboxes |
| Ubuntu Linux | Main ROS 2 development environment |
| ROS 2 | Robotics middleware and system integration |
| RViz | Visualizing maps, sensor scans and navigation |
| SLAM Toolbox or a suitable alternative | Mapping and localization |
| Nav2 or an equivalent navigation implementation | Goal-based navigation and obstacle handling |
| Python | Backend, utilities and integration |
| FastAPI and WebSockets | Web interface communication |
| HTML, CSS and JavaScript | Browser-based dashboard |
| Git and GitHub | Version control and portfolio |
| Mechanical CAD software | Chassis design and printable parts |

AutoCAD, AutoDesk Fusion 360

MATLAB and ROS 2 will have distinct responsibilities. MATLAB will help us understand and test algorithms; ROS 2 will form the primary integrated robotics stack unless early experiments reveal a better approach.

8. Web dashboard functionality

Manual mode

- Display a clear MANUAL indicator.

- Provide forward, reverse, left and right controls.

- Allow speed adjustment within safe limits.

- Send commands to the backend and robot controller.

- Stop the robot when the operator releases the control, where practical.

- Stop on communication timeout.

Autonomous mode

- Display the current map and estimated robot pose.

- Allow selection of a navigation goal.

- Display the planned route and current navigation status.

- Provide a cancel-goal or stop control.

- Report whether the goal was reached or navigation failed.

Monitoring

- Show Wi-Fi/robot connection status.

- Display available sensor measurements and battery status.

- Display errors and recent events.

- Record selected performance data for later analysis.

Manual and autonomous commands must not compete for motor control. Mode changes should be explicit, validated and safe.

9. Safety requirements

- Provide an accessible physical emergency-stop or motor-power cutoff.

- Enforce speed and acceleration limits.

- Use a command watchdog so the robot stops when control communication is lost.

- Reject invalid, stale or out-of-range movement commands.

- Stop or enter a safe state when critical sensor or navigation data becomes unavailable.

- Test in a controlled, clear area at low speed before introducing people or moving obstacles.

- Do not treat Wi-Fi connectivity or software obstacle detection as a substitute for a physical emergency stop.

10. Simulation and testing plan

Stage 1 — Algorithm simulation

Create simple rooms, walls and corridors. Simulate robot movement, sensor observations and navigation.

Stage 2 — Mapping and localization

Test SLAM and localization in several layouts. Investigate the effect of wheel errors, sensor noise and imperfect motion estimates.

Stage 3 — Path planning

Implement or configure a global planner. Compare planned paths and actual simulated motion.

Stage 4 — Obstacle handling

Introduce static and moving obstacles. Evaluate stopping behaviour, route replanning and navigation failures.

Stage 5 — Mechanical design

Design the chassis around the chosen components. Verify clearances, mounting points, centre of gravity, wheel dimensions and LiDAR visibility before printing.

Stage 6 — Hardware integration

Validate each subsystem separately: motor movement, encoder counts, LiDAR scans, odometry, map generation and autonomous navigation.

Stage 7 — Dashboard integration

Test telemetry display, manual commands, goal selection, mode switching and communication-loss behaviour.

Stage 8 — Final evaluation

Repeat experiments in measured indoor layouts and document successes, failures, limitations and improvements.

11. Performance metrics

- Mapping quality against a measured reference layout.

- Localization error at known reference positions.

- Successful goal arrivals across repeated trials.

- Navigation time and actual travel distance.

- Difference between planned and actual path lengths.

- Obstacle-response and replanning success.

- Communication latency and disconnect recovery.

- Battery usage during representative tasks.

- Performance differences between simulation and the physical robot.

Initial acceptance targets should be chosen after baseline tests. For example, 9 successful goal arrivals in 10 controlled trials with zero collisions is a reasonable target to investigate, not a result to assume.

12. Preliminary timeline

| Phase | Indicative duration | Deliverable |
| --- | --- | --- |
| Research and architecture | 2–3 weeks | Scope, diagrams and requirements |
| MATLAB fundamentals and simulation | 3–5 weeks | Working simulated navigation experiments |
| Lab inventory and component selection | 1–2 weeks, partly parallel | Approved bill of materials |
| CAD and chassis fabrication | 2–3 weeks | Printed and assembled prototype |
| Motor control and sensor integration | 2–4 weeks | Reliable movement and sensor data |
| SLAM and autonomous navigation | 3–6 weeks | Mapping and goal-based navigation |
| Web dashboard and communication | 2–4 weeks, partly parallel | Manual control and live monitoring |
| Validation and documentation | 2–3 weeks | Test results and portfolio materials |

These phases overlap, and the overall duration depends on learning speed, hardware availability and integration difficulties. There is no need to force completion against an artificial submission deadline.

13. Division of responsibilities

Teammate A — Hardware and robot integration

- Hardware inventory and component compatibility.

- Electrical and power-system design.

- Motor and encoder interfacing.

- CAD, 3D printing and mechanical assembly.

- Sensor integration and physical testing.

Teammate B — Robotics software and simulation

- MATLAB experiments and simulation.

- ROS 2 learning and configuration.

- SLAM, localization and path planning.

- Navigation testing and performance analysis.

- Test scripts and experiment documentation.

Shared responsibilities

- System architecture and scope.

- Backend/frontend implementation and interface design.

- Hardware-software integration.

- Safety testing.

- GitHub repository, report, demonstration and future feature selection.

Responsibilities are initial ownership assignments, not strict boundaries. Both teammates should understand the overall architecture.

14. Portfolio deliverables

- Public or appropriately shareable GitHub repository.

- Architecture and communication diagrams.

- MATLAB simulation files and recorded results.

- CAD source files and printed chassis.

- Hardware bill of materials and wiring diagrams.

- ROS 2 packages and configuration.

- Backend and frontend source code.

- Demonstration video.

- Quantitative test results and performance plots.

- Technical documentation explaining design choices and limitations.

15. Open decisions

The following should remain undecided until the first research and inventory phase:

- Exact onboard computer and LiDAR model.

- ROS 2 distribution and operating-system setup.

- MATLAB-first versus ROS 2-first simulation strategy.

- Chassis dimensions, wheel diameter and motor specifications.

- Battery voltage and runtime target.

- Exact dashboard features for the first working version.

- Initial budget and purchase order.

- Whether advanced localization recovery becomes part of the final scope.

Immediate next step: study the system architecture and establish the lab inventory. Do not purchase the complete hardware set or finalize the chassis before these two tasks are completed.

3. Proposed hardware BOM and budget

Here is the section missing from the document. These are planning estimates for India, not a purchase order. For reference, current listings show the RPLIDAR A1M8 at roughly ₹8,000–₹14,000 depending on supplier and listing.

Assumption: we can borrow some components from the IoT lab and print the chassis ourselves.

| Component | Suggested specification | Estimated cost |
| --- | --- | --- |
| Main computer | Raspberry Pi 5, 4 GB or 8 GB, with cooling and storage | ₹9,000–₹15,000 |
| LiDAR | RPLIDAR A1M8 or compatible 2D scanner | ₹8,000–₹14,000 |
| Drive motors | Two encoder-equipped geared DC motors | ₹1,000–₹3,000 |
| Motor driver | Compatible dual-channel driver rated for the motors | ₹500–₹2,500 |
| Microcontroller | ESP32 development board | ₹300–₹700 |
| Ultrasonic sensors | 2–3 × HC-SR04 | ₹150–₹450 |
| Power system | Battery, suitable regulators, charger and protection | ₹2,000–₹4,000 |
| Mechanical parts | Filament, fasteners, wheels and caster | ₹500–₹1,500 |
| Wiring and safety | Connectors, switch, fuse and emergency-stop hardware | ₹500–₹1,000 |
| Contingency | Replacement parts and unexpected requirements | ₹1,500–₹3,000 |

Estimated total if purchasing everything

₹23,450–₹45,150

With lab components borrowed and the chassis printed in-house, the actual cash expenditure could be substantially lower. The computer and LiDAR will usually dominate the cost.

If we choose four motors instead

The two-motor BOM above is not valid unchanged for a four-motor robot. We would need to account for:

- Two additional motors, ideally with encoders.

- A motor driver or driver combination rated for the combined motor current.

- A revised chassis, wheel mounts and possibly larger battery.

- More encoder inputs and additional calibration.

- A revised budget allowance of approximately ₹1,500–₹4,000 or more, depending on the motors and driver.

The driver should be selected only after checking each motor's rated voltage and stall current. A small TB6612FNG module, for example, should not be assumed suitable for four larger geared motors.

For ultrasonic sensors, I would start with one front-facing sensor during basic hardware testing, then add two angled front sensors if experiments show that they improve near-field obstacle detection. We should test them individually and stagger measurements to reduce cross-interference.

1. Four-wheel drive and four motors: should we use them?

Option A — Two drive motors + caster

Best for learning SLAM

- Two independently controlled wheels.

- Simpler kinematics and odometry.

- Easier to calibrate and troubleshoot.

- Less wheel scrub during turns.

Option B — Four motors, four wheels

More mechanical complexity

- Two motors on the left and two on the right.

- Left and right sides are controlled independently.

- Better potential for payload capacity and stability.

- Turning introduces more wheel scrub, which can worsen odometry.

- Requires careful motor matching and encoder calibration.

My recommendation: keep four-wheel drive as a genuine option, but don't assume it is automatically better.

Four motors can still form a differential-drive robot if the two left wheels work together and the two right wheels work together. This is commonly called a skid-steer configuration. It is entirely feasible, but the robot's motion estimates can become less accurate when the wheels slip or scrub during turns.

Because our primary challenge is SLAM and localization—not rough-terrain driving—I would initially prefer two powered wheels and a caster. If the lab already has four suitable encoder-equipped motors, we can compare both designs in simulation before choosing.